《数字逻辑》实验报告

| 姓名 |  |  |  | 年级 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 学号 |  |  |  | 专业、班级 |  |  |
| 实验名称 | 实验十五 摩尔状态机序列检测器 |  |  |  |  |  |
| 实验时间 | 2023.12.12 |  | 实验地点 |  | DS3402 |  |
| 实验成绩 |  |  | 实验性质 |  | □验证性 □设计性 □综合性 |  |
| 教师评价： □算法/实验过程正确； □源程序/实验内容提交 □程序结构/实验步骤合理； □实验结果正确； □语法、语义正确； □报告规范； 评语： 评价教师签名（电子签名）： |  |  |  |  |  |  |
| 一、实验目的 通过实验，掌握有限状态机的设计方法，并用摩尔状态机设计实现“1101” 序列检测。 |  |  |  |  |  |  |
| 二、实验项目内容 1、设计“1101”序列检测的状态转换图。 2、并转串模块设计，利用移位寄存器原理，setd 端上升沿将 8 位输入数据 同时锁存到移位寄存器中,在后续的 8 个时钟周期内将 8 位数据逐次移位 输出。 3、调用并转串输出模块，使用 Verilog HDL 语言的行为描述方式实现一个 摩尔状态机，能检测一个 8 位的二进制数据中是否存在“1101”序列，如 果检测到该序列则指定的 LED 灯亮。 4、综合、实现、生成 bit 流，下载到开发板进行验证。 |  |  |  |  |  |  |
| 三、实验设计 摩尔状态机序列检测器： 设计原理： 状态表与状态图是用来表示同步时序电路的输入、输出、现态、次态之间转移关系的两种常用工具。如果同步时序电路的输出只与现态有关而与输入无关，则称该电路为Moore型电路。 状态转换图： 顶层模块： debkey:消抖模块 PISO:并入串出模块 SIPO:串入并出模块，输入至显示模块方便显示 Detector1101:1101检测器 divider:分频器 display7seg:7段数码管显示模块 schematic： |  |  |  |  |  |  |
| 四、实验过程或算法 顶层模块 top_Detector1101.v module top_Detector1101( input button,clk,rst,in, input [7:0]din, output flag, output wire[3:0]an, output wire[6:0]display ); reg [25:0]target = 50000; wire button_deb; wire dout; wire clk_div; wire [3:0]dout_display; //debkey debkey deb(.clk(clk),.rst(rst),.key_in(button),.key_out(button_deb)); //PISO PISO P(.clk(button_deb),.rst(rst),.in(in),.din(din),.dout(dout)); //SIPO SIPO S(.clk(button_deb),.rst(rst),.din(dout),.dout(dout_display)); //Detector1011 Detector1101 D(.clk(button_deb),.rst(rst),.din(dout),.dout(flag)); //divider divider d(.clk(clk),.rst(rst),.target(target),.clk_div(clk_div)); //display7seg display7seg dis(.clk(clk_div),.data3(dout_display[3]),.data2(dout_display[2]),.data1(dout_display[1]),.data0(dout_display[0]),.an(an),.display(display)); endmodule 消抖模块 debkey.v module debkey( input clk, input rst, input key_in, output key_out ); parameter T100Hz = 249999; integer cnt_100Hz; reg clk_100Hz; always @(posedge clk) begin if(rst) cnt_100Hz<=32'b0; else begin cnt_100Hz<=cnt_100Hz+1'b1; if(cnt_100Hz==T100Hz) begin cnt_100Hz<=32'b0; clk_100Hz<=~clk_100Hz; end end end reg[2:0]key_rrr,key_rr,key_r; always @(posedge clk_100Hz) begin if(rst) begin key_rrr<=1'b1; key_rr<=1'b1; key_r<=1'b1; end else begin key_rrr<=key_rr; key_rr<=key_r; key_r<=key_in; end end assign key_out = key_rrr&key_rr&key_r; endmodule 并入串出模块 PISO.v //并入串出 module PISO( input clk,rst,in, input [7:0]din, output reg dout ); reg [7:0] data; //data&dout always @(posedge clk or posedge rst or posedge in) begin if(rst) {dout,data} <= 9'b0; else begin if(in) begin data<=din; dout<=1'b0; end else {dout,data[7:0]} <= {data[7:0],1'b0}; end end endmodule 串入并出模块 SIPO.v //串入并出 module SIPO( input clk,rst, input din, output reg [3:0]dout ); always @(posedge clk) begin if(rst) dout <= 4'b0; else begin dout <= {dout,din}; end end endmodule 1101序列检测模块 Detector1101.v module Detector1101( input clk,rst,din, output reg dout ); reg[2:0] state,next_state; parameter S0 = 3'b000,S1=3'b001,S2=3'b010,S3=3'b011,S4=3'b100; always @(posedge clk or posedge rst) begin if(rst) state<=3'b000; else state<=next_state; end always @(state or din) begin case(state) S0:begin if(din==1) next_state=S1; else next_state=S0; end S1:begin if(din==1) next_state=S2; else next_state=S0; end S2:begin if(din==1) next_state=S2; else next_state=S3; end S3:begin if(din==1) next_state=S4; else next_state=S0; end S4:begin if(din==1) next_state=S2; else next_state=S0; end default: next_state=S0; endcase end always @(state) begin case(state) S4: dout = 1'b1; default:dout = 1'b0; endcase end endmodule 分频器模块 divider.v module divider( input clk,rst, input [15:0] target, output reg clk_div ); reg [15:0] counter; always @(posedge clk) begin if(rst) begin counter <= 0; clk_div <= 0; end else if(counter==target) begin counter <= 0; clk_div <= ~clk_div; end else counter <= counter+1; end endmodule 七段数码管显示模块 display7seg.v module display7seg( input clk, input [3:0]data3,data2,data1,data0, output reg[3:0]an, output reg[6:0]display ); reg [1:0] count; always @(posedge clk) begin if(count == 'b11) count <= 0; else count <= count +'b1; end always @(posedge clk) begin case(count) 2'b00: an <= 4'b1110; 2'b01: an <= 4'b1101; 2'b10: an <= 4'b1011; 2'b11: an <= 4'b0111; endcase end always @(posedge clk) begin case(count) 2'b00: case (data0) 4'b0000:display = 7'b0000001; 4'b0001:display = 7'b1001111; 4'b0010:display = 7'b0010010; 4'b0011:display = 7'b0000110; 4'b0100:display = 7'b1001100; 4'b0101:display = 7'b0100100; 4'b0110:display = 7'b0100000; 4'b0111:display = 7'b0001111; 4'b1000:display = 7'b0000000; 4'b1001:display = 7'b0000100; endcase 2'b01: case (data1) 4'b0000:display = 7'b0000001; 4'b0001:display = 7'b1001111; 4'b0010:display = 7'b0010010; 4'b0011:display = 7'b0000110; 4'b0100:display = 7'b1001100; 4'b0101:display = 7'b0100100; 4'b0110:display = 7'b0100000; 4'b0111:display = 7'b0001111; 4'b1000:display = 7'b0000000; 4'b1001:display = 7'b0000100; endcase 2'b10: case (data2) 4'b0000:display = 7'b0000001; 4'b0001:display = 7'b1001111; 4'b0010:display = 7'b0010010; 4'b0011:display = 7'b0000110; 4'b0100:display = 7'b1001100; 4'b0101:display = 7'b0100100; 4'b0110:display = 7'b0100000; 4'b0111:display = 7'b0001111; 4'b1000:display = 7'b0000000; 4'b1001:display = 7'b0000100; endcase 2'b11: case (data3) 4'b0000:display = 7'b0000001; 4'b0001:display = 7'b1001111; 4'b0010:display = 7'b0010010; 4'b0011:display = 7'b0000110; 4'b0100:display = 7'b1001100; 4'b0101:display = 7'b0100100; 4'b0110:display = 7'b0100000; 4'b0111:display = 7'b0001111; 4'b1000:display = 7'b0000000; 4'b1001:display = 7'b0000100; endcase endcase end endmodule |  |  |  |  |  |  |
| 五、实验过程中遇到的问题及解决情况 1.状态机的三段式写法中，时序逻辑和组合逻辑没有分清，导致仿真结果不正确 解决方案：将输出赋值由时序逻辑改为组合逻辑，仿真结果正确。 |  |  |  |  |  |  |
| 六、实验结果及分析和（或）源程序调试过程 1. 并入串出模块 仿真文件：PISO_tb.v module PISO_tb( ); reg clk,rst,in; wire dout; reg[7:0]din; initial begin clk = 0; forever #5 clk=~clk; end PISO P(.clk(clk),.rst(rst),.in(in),.din(din),.dout(dout)); initial begin #10 rst = 1; #10 rst = 0;in = 1;din = 8'b10110110; #10 in = 0; #100 rst = 0;in = 1;din = 8'b11111111; #10 in = 0; end endmodule PISO 仿真波形图： 2. 传入并出模块 仿真文件SIPO_tb.v module SIPO_tb( ); reg clk,rst,in,din; wire [3:0]dout; initial begin clk = 0; forever #5 clk=~clk; end SIPO S(.clk(clk),.rst(rst),.din(din),.dout(dout)); initial begin #10 rst = 1; repeat(50) begin #10 rst = 0;in = 1;din = $random; end end endmodule SIPO 仿真波形图： 3.Detector1101模块 1101序列检测仿真文件：Detector1101_tb.v module Detector1101_tb( ); reg clk,rst,din; wire dout; reg [3:0]count = 0; initial begin clk = 0; forever begin #5 clk = ~clk; end end Detector1101 D(.clk(clk),.rst(rst),.din(din),.dout(dout)); initial begin rst = 1; #10 rst = 0; end always @(posedge clk) begin count <= count + 1; end always @(*) begin case (count) 4'd0:din = 1; 4'd1:din = 1; 4'd2:din = 0; 4'd3:din = 1; default: din = 0; endcase end endmodule 仿真波形图： 4. 约束文件： set_property IOSTANDARD LVCMOS33 [get_ports {display[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {display[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {display[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}] set_property IOSTANDARD LVCMOS33 [get_ports button] set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {display[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[6]}] set_property IOSTANDARD LVCMOS33 [get_ports rst] set_property IOSTANDARD LVCMOS33 [get_ports {display[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {display[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[1]}] set_property IOSTANDARD LVCMOS33 [get_ports in] set_property IOSTANDARD LVCMOS33 [get_ports clk] set_property IOSTANDARD LVCMOS33 [get_ports {display[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {din[7]}] set_property IOSTANDARD LVCMOS33 [get_ports flag] set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}] set_property PACKAGE_PIN U18 [get_ports rst] set_property PACKAGE_PIN U16 [get_ports flag] set_property PACKAGE_PIN R2 [get_ports {din[7]}] set_property PACKAGE_PIN T1 [get_ports {din[6]}] set_property PACKAGE_PIN U1 [get_ports {din[5]}] set_property PACKAGE_PIN W2 [get_ports {din[4]}] set_property PACKAGE_PIN R3 [get_ports {din[3]}] set_property PACKAGE_PIN T2 [get_ports {din[2]}] set_property PACKAGE_PIN T3 [get_ports {din[1]}] set_property PACKAGE_PIN V2 [get_ports {din[0]}] set_property PACKAGE_PIN W5 [get_ports clk] set_property PACKAGE_PIN V16 [get_ports in] set_property PACKAGE_PIN W19 [get_ports button] set_property PACKAGE_PIN W4 [get_ports {an[3]}] set_property PACKAGE_PIN V4 [get_ports {an[2]}] set_property PACKAGE_PIN U4 [get_ports {an[1]}] set_property PACKAGE_PIN U2 [get_ports {an[0]}] set_property PACKAGE_PIN W7 [get_ports {display[6]}] set_property PACKAGE_PIN W6 [get_ports {display[5]}] set_property PACKAGE_PIN U8 [get_ports {display[4]}] set_property PACKAGE_PIN V8 [get_ports {display[3]}] set_property PACKAGE_PIN U5 [get_ports {display[2]}] set_property PACKAGE_PIN V5 [get_ports {display[1]}] set_property PACKAGE_PIN U7 [get_ports {display[0]}] 摩尔状态序列机检测器上板演示效果： 输入序列“11011010” 连续按移位键进行“1101”序列的检测，一旦检测到提示灯就会亮起 如果继续移位，序列不再是“1101”，提示灯熄灭 在按下重置键后，数码管显示数字归零，至此所有功能验证完毕 七、小组分工情况说明 ：参与实验原理分析，实验设计，设计文件编写。撰写实验报告。 ：参与实验原理分析，实验设计，仿真文件编写与测试。撰写实验报告。 ：参与实验原理分析，实验设计，综合，生成bit流，并下载至开发板验证功能是否完整，有无错误，进行修改。撰写实验报告。 |  |  |  |  |  |  |